# REQUIREMENTS

## MOTION CONTROLLER

This software is intended to be used with a DESKCNC controller board.

http://deskcnc.com/controller2nd.html

It was developed to drive a MADELL PX-3702 pick and place machine.

The controller CAN be used with a USB to SERIAL converter on computers with no serial ports.

## VIDEO CAMERAS

TWO USB video cameras with MANUAL FOCUS lenses. (auto focus cameras will have problems with vibration). The camera cables can be extended with USB ACTIVE EXTENSION CABLES.

#### Down looking camera

This camera is used to locate CALIBRATION MARKS and PCB fiducial marks. Lighting for this camera needs to be consistent but is not critical.

#### Up looking camera

This camera is used to align and centre components before placing.

This camera should be located at the BOARD HEIGHT to eliminate vertical camera errors.

Lighting is critical for this camera. I use a 9 LED white ring light on my up-looking camera.

Turning OFF the auto exposure feature of the up-looking camera (if possible) may also provide better results.

## MACHINE SETUP

This software treats the HOME position as the FRONT LEFT corner (the corner with the home switches)

With the machine oriented this way the camera image to be normal orientation (not upside down!).

Both the up-looking and down-looking cameras MUST be set to "NORMAL" and not "MIRRORED".

The HOME can use LIMIT SWITCHES, or both LIMIT SWITCHES AND VISUAL REFERENCES.

The IMAGE WIDTH is the width of the video picture in MM at the HEIGHT of the target object.

For the UP camera this will be when focused on the nozzle tip.

For the DOWN camera this will be when looking at the PCB. The HOME REFERENCE MARK(S) should be at the SAME HEIGHT as the PCB to allow the camera focus to be set to suit both.

The Correlation limit is the MINIMUM LIMIT for IMAGE MATCHING of the computer vision. Around 65% to 70% seems to work best for me.

The ENLARGEMENT value is the number of MM wider that the computer vision should try searching for the target.

The Angle test limit is the MAXIMUM error angle that the computer vision should try searching for ANGLULAR CORRECTION of the part.